| Test Name | Manually start pump when pump is not running [Est.Timer] |
| --- | --- |
| Use Case Tested: | Manually turn the pump on |
| Test Description: | This test verifies that the pump can be turned on using the manual on/off switch, and the estimated timer value is sent |
| Pre-conditions | - Pump Not Running; - System State of Pump: Not running; - ThingsBoard has not asked the pump to start running via manual operation. - Automation: Off. - Est. Timer: On. - Custom Timer: Off. |
| Post-conditions | - Running LED: On - Manual Switch: On - Transmit Status: Successful - Pump On with Est. Timer value |
| Notes: |  |

|  | - TEST STEP | EXPECTED TEST RESULTS |
| --- | --- | --- |
|  | - Set tank level sensor server attribute fillTimer to 2. | - Est. Fill Timer on dashboard shows 00h 02m. |
|  | - Use the Dash to send a manual pump on message. | - Switch changes from off to on. - Transmit status becomes “In-Transit”. - Event Log shows Manual On message. - A downlink command is scheduled with payload 0x03 0x00 0x02. |
|  | - Send a status message from pump controller. | - Pump controller sends a status message with pumpRunning: 0. - Dash still displays “In-Transit”. |
|  |  | - Pump controller receives command and switches pump on with a message giving the timer information. - Pump controller sends a status message with pumpRunning: 1. - Dash displays transmit status success. |
|  | - Wait 2 minutes | - Pump controller switches pump off, sends a status message with pumpRunning: 0. - Dashboard updates to show pump not running. |

| Test Data Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| [Data field 1] | [data set 1 input value for field 1] |  |  |  |  |
| [Data field 2] | [data set 1 input value for field 2] |  |  |  |  |
| [Data field 3] | [data set 1 input value for field 3] |  |  |  |  |

Results

05/09/2020

SUCCESS!

Manual On sent. Received with correct timer value. Dash updated.

9/9/2020 – On site test at OAI.

Success.

Step 1.

Step 2.

Steps 3 & 4.

11:17:06.701 -> Send operator requested status message.

11:17:06.701 -> LoRaWAN will encode and send this message: {'pumpRunning':0,'boreLowLevel':0,'softStartFail':0,'pumpOverload':0,'controllerRestart':0,'highPressure':0,'noFlow':0}

11:17:06.701 -> Sending status byte: 00

11:17:07.796 -> EV_TXCOMPLETE (includes waiting for RX windows)

11:17:07.796 -> Received reply with 3 bytes:

11:17:07.796 -> 03 00 02

11:17:07.796 -> callback got data 3

11:17:07.796 -> Switched pump on.

11:17:07.796 -> Starting one-shot timer.

11:17:07.796 -> Timer in minutes: 2

11:17:07.796 -> Sending status due to state change.

11:17:07.796 -> LoRaWAN will encode and send this message: {'pumpRunning':1,'boreLowLevel':0,'softStartFail':0,'pumpOverload':0,'controllerRestart':0,'highPressure':0,'noFlow':0}

11:17:07.796 -> Sending status byte: 01

Step 5.

11:19:07.802 -> Pump run timer has expired. Turning pump off.

11:19:07.802 -> Switched pump off.

11:19:07.802 -> Sending status due to state change.

11:19:07.802 -> LoRaWAN will encode and send this message: {'pumpRunning':0,'boreLowLevel':0,'softStartFail':0,'pumpOverload':0,'controllerRestart':0,'highPressure':0,'noFlow':0}

11:19:07.802 -> Sending status byte: 00

11:19:09.907 -> EV_TXCOMPLETE (includes waiting for RX windows)